AI-POWERED SMART WASTE SEGREGATION SYSTEM

SOFTWARE ARCHITECTURE, DIGITAL TWIN DASHBOARD & TELEMETRY REPORT

| System Type: Industrial IoT Digital Twin & Safety Control | Frontend Stack: React 19 + Vite 8 + Tailwind CSS |
| --- | --- |
| Cloud IoT Platform: ThingSpeak REST API (Channel #3470506) | Telemetry Rate: 10-Second Continuous Heartbeat Poller |

# 1. Executive Summary & Software Scope

This software documentation presents the complete digital architecture, frontend framework design, real-time telemetry ingestion pipelines, safety state evaluation engine, and analytics visualization platform for the AI-Powered Smart Waste Segregation Bin — Digital Twin System. Built as a production-grade Industrial IoT (IIoT) control center, the software functions as a high-fidelity Digital Twin that renders live fluid capacity levels across multiple physical bin compartments, monitors critical safety parameters (flame, combustible gas/smoke, and ferrous metal proximity), and automatically alerts system administrators when hazardous thresholds are exceeded.

[NOTE] Edge AI vs. Digital Twin Telemetry Isolation Guarantee The software architecture enforces strict decoupling between physical hardware intelligence and web visualization. Optical image classification (PET/Plastic, Dry/Recyclable, Organic) is executed 100% locally on the ESP32-CAM microcontroller edge hardware. The React Digital Twin Dashboard does NOT run synthetic inferences or artificial predictions; it serves exclusively as an authoritative, high-fidelity monitoring and safety control station consuming live cloud telemetry stream.

# 2. System Architecture & Data Flow

The complete end-to-end data pipeline spans four primary operational layers: Edge Microcontroller Layer, Cloud IoT Data Gateway, React Application Service Layer, and Neumorphic Digital Twin UI Layer.

+-----------------------------------------------------------------------------------+
|                                 SMART WASTE BIN HARDWARE                          |
|  +---------------------------+                +---------------------------------+  |
|  |     ESP32-CAM (Edge AI)   |                | Physical Sensors (HC-SR04, MQ2, |  |
|  | Local Vision Classification|                | Flame Detector, Proximity)      |  |
|  +-------------+-------------+                +----------------+----------------+  |
+----------------|-----------------------------------------------+------------------+
                 |                                               |
                 +-----------------------+-----------------------+
                                         |
                                         v
                            +--------------------------+
                            | ESP32 Main IoT Controller|
                            +------------+-------------+
                                         | HTTP POST (JSON / Form)
                                         v
                            +--------------------------+
                            |   ThingSpeak Cloud IoT   |
                            |   (REST Field Channels)  |
                            +------------+-------------+
                                         | HTTP GET (10s Polling Hook)
                                         v
                            +--------------------------+
                            | React 19 Digital Twin    |
                            | Dashboard (Vite Frontend)|
                            +--------------------------+

# 3. Software Technology Stack & Library Inventory

The application is constructed using a modern React 19 ecosystem bundled with Vite 8. Below is the comprehensive software dependency matrix:

| Package Name | Version | Role / Purpose | Architectural Scope |
| --- | --- | --- | --- |
| React | ^19.2.8 | Core Component Engine & Declarative UI State | Frontend Framework |
| Vite | ^8.2.2 | Lightning-fast HMR Build Tool & Module Bundler | Build System |
| Tailwind CSS | ^3.4.17 | Utility-First CSS with Dark Neumorphic Tokens | Styling Engine |
| Axios | ^1.20.0 | Promise-based HTTP Client for ThingSpeak REST API | Data Ingestion |
| Recharts | ^3.10.1 | SVG Time-Series Analytics & Telemetry Line/Area Charts | Data Visualization |
| Lucide React | ^1.38.0 | Industrial IoT SVG Vector Icon Set | User Interface |
| Framer Motion | ^13.1.1 | Smooth Micro-animations & Fluid Container Transitions | UI Animation |
| Oxlint | ^1.79.0 | High-performance JS/JSX Code Quality & Linter | Code Maintenance |

# 4. Cloud IoT Communication & ThingSpeak Schema

Communication between physical bin sensors and the software dashboard is mediated through the MathWorks ThingSpeak IoT Cloud platform (Channel ID: 3470506). The frontend queries the REST channels asynchronously every 10 seconds to retrieve incoming sensor telemetry feeds.

| ThingSpeak Field | Physical Sensor Name | Target Metric | Valid Range / Format | UI Destination |
| --- | --- | --- | --- | --- |
| Field 1 | Ultrasonic Sensor 1 | Bin 01 Fill Level | 0% - 100% (or distance cm) | BIN 01 — PET / PLASTIC |
| Field 2 | Ultrasonic Sensor 2 | Bin 02 Fill Level | 0% - 100% (or distance cm) | BIN 02 — DRY / RECYCLABLE |
| Field 3 | Ultrasonic Sensor 3 | Bin 03 Fill Level | 0% - 100% (or distance cm) | BIN 03 — ORGANIC / GENERAL |
| Field 4 | Optical Flame Sensor | Fire / Spark Hazard | 0 = Safe, 1 = Flame Alert | Flame Sensor & Emergency Banner |
| Field 5 | Gas / Smoke (MQ-2) | Air Quality / Smoke PPM | 0 - 1000+ PPM | Radial Gas Gauge (<300 Safe, >=800 Danger) |
| Field 6 | Inductive Proximity | Ferrous Metal Detection | 0 = No Metal, 1 = Metal | Proximity Metal Indicator Card |

## ThingSpeak REST API Integration Endpoints

1. Fetch Latest Telemetry Entry:
   GET https://api.thingspeak.com/channels/3470506/feeds/last.json?api_key=W2K6VI4WUKVWX3N3
2. Fetch Historical Feeds (40 frames for time-series charts):
   GET https://api.thingspeak.com/channels/3470506/feeds.json?api_key=W2K6VI4WUKVWX3N3&results=40

# 5. Software Logic, Algorithms & Mathematical Models

The software framework encapsulates several dedicated algorithmic models within `sensorUtils.js` and `alertUtils.js` to process raw telemetry values into actionable operational states.

## Algorithm 1: Ultrasonic Fill Level Calibration Formula

When operating under RAW_DISTANCE calibration mode, the fill percentage is calculated using linear inverse transformation bounded between empty (40cm) and full (5cm) thresholds:

Fill % = min(100, max(0, ((Empty_Distance - Current_Distance) / (Empty_Distance - Full_Distance)) * 100))

## Algorithm 2: Tri-State Global Safety System Decision Engine

The system evaluates environmental safety parameters simultaneously using a hierarchical priority rule matrix:

• EMERGENCY STATE (Priority 1): Triggered if Flame Sensor == 1 (Field 4) OR Gas Concentration >= 800 PPM (Field 5). Result: Halts sorting, plays animated emergency alert overlay.

• WARNING STATE (Priority 2): Triggered if Any Bin Fill Level >= 80% OR Gas Concentration is between 300 PPM and 799 PPM. Result: Displays warning badge and prepares collection dispatch.

• NORMAL STATE (Priority 3): Triggered when Flame == 0 AND Gas < 300 PPM AND All Bins < 80%. Result: Nominal continuous monitoring.

## Algorithm 3: Non-Duplicative State Transition Logger Engine

To prevent redundant notification floods during continuous 10-second polling, `alertUtils.js` tracks previous state hashes. A new event entry is logged ONLY when a genuine threshold boundary is crossed (e.g., transition from SAFE to HIGH smoke, or NORMAL to ALMOST FULL bin capacity). Event logs are stored in a ring-buffer capped at 50 historical entries.

# 6. UI Component Architecture & Digital Twin Visualizers

The user interface follows a modular, industrial control center design pattern using Neumorphic and Glassmorphic aesthetics. Key components include:

▪ Header.jsx: Top navigation bar displaying live pulse indicator, last sync timestamp, channel ID badge, and manual telemetry sync button.

▪ SystemStatus.jsx: Global banner rendering System State (NORMAL, WARNING, EMERGENCY) and high-priority fire/gas warning alerts.

▪ KPIGrid.jsx: Top metrics grid featuring 7 cards: System State, Active Bins, Highest Fill %, Fire Alert, Gas Concentration, Metal Detection, and Sensor Signal Health.

▪ DigitalTwinBin.jsx: Master 3D physical bin cylinder visualization with dynamic fluid wave meniscus, height percentages, HC-SR04 bracket graphic, and fill markings (25%, 50%, 75%, 80%, 95%, 100%).

▪ SafetyPanel.jsx & Components: Dedicated multi-sensor safety card container encapsulating FlameSensor.jsx, SmokeGauge.jsx (animated radial SVG gas meter), and MetalDetection.jsx.

▪ SensorHealth.jsx: Diagnostic telemetry matrix assessing active vs. inactive fields across all physical sensors.

▪ CollectionStatus.jsx: Waste logistics planner providing automated collection recommendations and priority dispatch statuses.

▪ AlertsPanel.jsx: Event log timeline with severity filter tabs (All, Critical, Warning, Info).

▪ FillLevelChart.jsx & SmokeChart.jsx: Recharts historical telemetry visualization rendering fill % and smoke PPM time series across 40 frames.

# 7. Source Code Directory Structure

smart-waste/
├── .env                         # ThingSpeak Channel ID & Read Key configuration
├── index.html                   # HTML entrypoint with Inter/JetBrains fonts
├── package.json                 # Frontend dependencies and Vite build scripts
├── postcss.config.js            # PostCSS configuration
├── tailwind.config.js           # Custom dark industrial neumorphic theme tokens
└── src/
    ├── config/
    │   └── sensorConfig.js      # Sensor thresholds, mode & bin metadata
    ├── services/
    │   └── thingspeak.js        # Axios REST client & feed normalization
    ├── utils/
    │   ├── sensorUtils.js       # Fill calculation, status & parsing utilities
    │   └── alertUtils.js        # Non-duplicative state transition logger
    ├── hooks/
    │   └── useThingSpeak.js     # 10s polling custom hook with window focus pause
    ├── components/
    │   ├── Header.jsx           # Live pulse indicator, sync button, channel ID
    │   ├── SystemStatus.jsx     # Emergency banner & global status bar
    │   ├── KPIGrid.jsx          # 7 key operational metric summary cards
    │   ├── StatusCard.jsx       # Reusable glassmorphic container card
    │   ├── DigitalTwin.jsx      # Master 3D-styled bin container wrapper
    │   ├── DigitalTwinBin.jsx   # Animated fluid cylinder with level markings
    │   ├── SafetyPanel.jsx      # Multi-sensor safety container
    │   ├── FlameSensor.jsx      # Optical fire hazard indicator
    │   ├── SmokeGauge.jsx       # Animated radial SVG gas gauge (0-1000 PPM)
    │   ├── MetalDetection.jsx   # Inductive proximity sensor state card
    │   ├── SensorHealth.jsx     # Telemetry diagnostics grid for all 6 fields
    │   ├── CollectionStatus.jsx # Automated logistics recommendation planner
    │   ├── AlertsPanel.jsx      # Real-time event log with severity filtering
    │   ├── FillLevelChart.jsx   # Recharts fill % historical time series
    │   ├── SmokeChart.jsx       # Recharts gas/smoke historical time series
    │   ├── LoadCellPlaceholder.jsx # Future hardware weight expansion card (-- g)
    │   ├── PowerPlaceholder.jsx    # Future power telemetry card (-- W, -- V)
    │   └── EdgeAINote.jsx       # ESP32-CAM Edge AI architecture diagram
    ├── pages/
    │   └── Dashboard.jsx        # Complete assembled dashboard layout
    ├── App.jsx                  # Root React application component
    ├── main.jsx                 # Vite application entrypoint
    └── index.css                # Tailwind directives & industrial dark styles

# 8. Setup, Build & Deployment Manual

Follow these commands to configure, build, and deploy the software dashboard:

• Step 1: Environment Setup:
   Create a .env file in the project root:
   VITE_THINGSPEAK_CHANNEL_ID=3470506
   VITE_THINGSPEAK_READ_API_KEY=W2K6VI4WUKVWX3N3

• Step 2: Install Node Dependencies:
   Execute in shell: npm install

• Step 3: Launch Local Development Server:
   Execute in shell: npm run dev (Launches Vite dev server on http://localhost:5173)

• Step 4: Execute Production Build:
   Execute in shell: npm run build (Outputs optimized static files to /dist)

• Step 5: Preview Production Bundle:
   Execute in shell: npm run preview